# Design of a Simple RISC Processor – Full Project Report

### CSC 320 — Computer Organization

### Lebanese American University

### Fall 2025

# 1. Introduction

This report presents the complete design of a simple RISC processor as required by the course project. The processor supports arithmetic operations, shift operations, and specialized memory access instructions involving shifted register offsets. It follows a four‑stage control flow: Fetch, Decode, Execute, and Write‑Back, and uses separate instruction memory and data memory.

The processor contains 8 general‑purpose registers (R0–R7) and a Program Counter (PC). All instructions are encoded into a 16‑bit instruction word.

# 2. Register Identifiers (Q1)

The processor includes 8 registers: R0–R7. Each register is uniquely identified using a 3‑bit binary code.

| Register | Binary Identifier |
| --- | --- |
| R0 | 000 |
| R1 | 001 |
| R2 | 010 |
| R3 | 011 |
| R4 | 100 |
| R5 | 101 |
| R6 | 110 |
| R7 | 111 |

The PC register is not encoded in instruction fields and is managed separately.

# 3. Instruction Size (Q2)

### Chosen instruction size: 16 bits

This size is the smallest power‑of‑two value that can hold all required fields.

### Bit Breakdown

- Opcode: 3 bits (8 instructions → 2³ = 8)

- Destination register: 3 bits

- Source/Base register: 3 bits

- Offset register: 3 bits

- Shift amount: 4 bits

Total = 3 + 3 + 3 + 3 + 4 = 16 bits

This makes instruction memory word‑addressable and simplifies PC incrementing.

# 4. Register Size (Q3)

All registers are 32 bits wide.

Justification: - Provides compatibility with 32‑bit word operations. - Supports large data memory addressing. - Matches typical RISC educational architectures.

# 5. Maximum Data Memory Capacity (Q4)

Data memory is word‑addressable, with each word = 4 bytes.

Since addresses come from 32‑bit registers: - Maximum number of addressable words = 2³² - Total size = 2³² words × 4 bytes/word = 2³⁴ bytes = 16 GB

Thus, the processor can theoretically support up to 16 gigabytes of data memory.

# 6. Instruction Structure and Field Sizes (Q5)

The processor uses two formats, both 16 bits wide.

## 6.1 Format A — ALU Instructions

|15..13|12..10|9..7 |6..4 |3..0 |
|Opcode|Rdest |Rsrc1|Rsrc2|Sh   |

Used for: - ADD - SUB - SHL - SHR

Fields: - Opcode (3 bits) - Rdest (3 bits) - Rsrc1 (3 bits) - Rsrc2 (3 bits) - Sh (4 bits) — immediate shift amount

## 6.2 Format B — Memory Instructions (Load/Store with shifted offset)

|15..13|12..10|9..7 |6..4 |3..0 |
|Opcode|Rdest |Rbase|Roff |Sh   |

Used for: - ShiftLeftLoad - ShiftRightLoad - ShiftLeftStore - ShiftRightStore

Fields: - Opcode (3 bits) - Rdest (3 bits) → destination for loads, source for stores - Rbase (3 bits) - Roff (3 bits) → offset register - Sh (4 bits) → shift amount

Address calculation:

Address = Rbase + (Roff << Sh)    for left-shift instructions
Address = Rbase + (Roff >> Sh)    for right-shift instructions

# 7. Instruction Syntax & Opcodes (Q6)

The processor supports eight instructions, each with a dedicated opcode.

| Opcode | Binary | Instruction | Syntax | Function |
| --- | --- | --- | --- | --- |
| 0 | 000 | ADD | ADD Rd, Rs1, Rs2 | Rd = Rs1 + Rs2 |
| 1 | 001 | SUB | SUB Rd, Rs1, Rs2 | Rd = Rs1 – Rs2 |
| 2 | 010 | SHL | SHL Rd, Rs, Sh | Rd = Rs << Sh |
| 3 | 011 | SHR | SHR Rd, Rs, Sh | Rd = Rs >> Sh |
| 4 | 100 | ShiftLeftLoad | LL Rd, Rbase, Roff, Sh | Rd = MEM[Rbase + (Roff << Sh)] |
| 5 | 101 | ShiftRightLoad | LR Rd, Rbase, Roff, Sh | Rd = MEM[Rbase + (Roff >> Sh)] |
| 6 | 110 | ShiftLeftStore | SL Rs, Rbase, Roff, Sh | MEM[…] = Rs |
| 7 | 111 | ShiftRightStore | SR Rs, Rbase, Roff, Sh | MEM[…] = Rs |

# 8. Program Counter Increment (Q7)

Instruction memory is word-addressable, and each instruction is 1 word (16 bits).

Therefore: ### PC = PC + 1 after every instruction fetch.

# 9. Datapath Design (Q8)

Below are text‑based block diagrams representing the datapath. These correspond to the required processor behavior.

## 9.1 Datapath Components - Program Counter (PC) - Instruction Memory (IMEM) - Instruction Register (IR) - Register File (R0–R7) - Shift Unit (left/right shift by immediate) - ALU (add/sub) - Address Adder (for memory instructions) - Data Memory (DMEM) - Write‑back MUX - Control Unit - Various multiplexers

## 9.2 Datapath for ALU Instructions

+---------+
 PC -------> |  IMEM   | -----> IR -------+
             +---------+                  |
                                           v
                             +-----------------------+
                             |      Control Unit     |
                             +-----------------------+
                                  |   |   |   
     +----------------+           |   |   |---------- RegWrite
     |  Register File |<----------+   |-------------- ALUOp
     +----------------+               +-------------- Shift signals
        ^     ^     ^
        |     |     |
     Rdest  Rsrc1  Rsrc2
        |     |     |
        |     |     +---------> ALU -------+
        |                     ^            |
        |                     |            |
        +---------------------+            |
                                           v
                                    Write-back MUX
                                           |
                                           v
                                    Register File

## 9.3 Datapath for Load Instructions

PC -> IMEM -> IR -> Control Unit

 Register File (Rbase, Roff) ---------+
                                      |
                                      v
                                +-----------+
                                | Shift Unit|
                                +-----------+
                                      |
                                      v
                             +-------------------+
                             | Address Adder     |
                             | Rbase + shifted   |
                             +-------------------+
                                      |
                                      v
                                  Data Memory -----> Write-back MUX -> Register File

## 9.4 Datapath for Store Instructions

Store value (Rs)
                   |
                   v
 Register File ----+-------------------------------+
                   |                               |
                   v                               v
                Shift Unit -> Address Adder -> Data Memory (write)

# 10. Control Flow Stages

Each instruction goes through:

### 1. Fetch

- IR ← IMEM[PC]

- PC ← PC + 1

### 2. Decode

- Opcode is decoded

- Register file outputs Rdest, Rsrc1, Rsrc2 OR Rbase, Roff

- Shift amount extracted

### 3. Execute

- ALU operations for arithmetic

- Shift + address addition for memory instructions

### 4. Write‑Back

- ALU or DMEM output is written into Rdest (for loads)

- Stores do not write to registers

# 11. Example Instruction Executions

## 11.1 Example: LL R1, R2, R3, 2

Address = R2 + (R3 << 2)
R1 = MEM[Address]

## 11.2 Example: ADD R4, R1, R2

R4 = R1 + R2

# 12. Demo Requirements (for submission)

Your demo must show: - Datapath with highlighted signal values - Flow of at least one ALU and one memory instruction - Example of shifted offset calculation - Register file before/after execution

# 13. Conclusion

This report fully defines the processor architecture, instruction formats, opcodes, datapath, and execution behavior. It meets all requirements outlined in the CO Project (Fall 2025) assignment.

# End of Report